实验4 路由器OSPF动态路由配置实验

班级：                  姓名：                学号：

上机时间：              上机地点：             指导老师：

一、实验目标

掌握OSPF协议的配置方法；

掌握查看通过动态路由协议OSPF学习产生的路由；

熟悉广域网线缆的链接方式。

二、实验背景

假设校园网通过一台三层交换机连到校园网出口路由器上，路由器再和校园外的另一台路由器连接。现要做适当配置，实现校园网内部主机与校园网外部主机之间的相互通信。为了简化网管的管理维护工作，学校决定采用OSPF协议实现互通。

三、技术原理

OSPF开放式最短路径优先协议，是目前网路中应用最广泛的路由协议之一。属于内部网管路由协议，能够适应各种规模的网络环境，是典型的链路状态协议。OSPF路由协议通过向全网扩散本设备的链路状态信息，使网络中每台设备最终同步一个具有全网链路状态的数据库，然后路由器采用SPF算法，以自己为根，计算到达其他网络的最短路径，最终形成全网路由信息。

四、实验步骤

新建packet tracer拓扑图。

（1）在本实验中的三层交换机上划分VLAN10和VLAN20，其中VLAN10用于连接校园网主机，VLAN20用于连接R1。

（2）路由器之间通过V35电缆通过串口连接，DCE端连接在R1上，配置其时钟频率64000。

（3）主机和交换机通过直连线，主机与路由器通过交叉线连接。

（4）在S3560上配置OSPF路由协议。

（5）在路由器R1、R2上配置OSPF路由协议。

（6）将PC1、PC2主机默认网关设置为与直连网路设备接口IP地址。

（7）验证PC1、PC2主机之间可以互相通信。

五、实验设备及拓扑

PC 2台；Switch_3560 1台；Router-PT 2台；直连线；交叉线；DCE串口线

六、实验过程

PC1

IP:			192.168.1.2

Submask:		255.255.255.0

Gateway:		192.168.1.1

PC2

IP:			192.168.2.2

Submask:		255.255.255.0

Gateway:		192.168.2.1

S3560

en

conf t

hostname S3569

vlan 10

exit

vlan 20

interface fa 0/10

switchport access vlan 10

exit

int fa 0/20

switchport access valn 20

exit

interface valn 10

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface vlan 20

ip address 192.168.3.1 255.255.255.0

no shutdown

end

show ip route

conf t

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 192.168.3.0 0.0.0.255 area 0

end

show ip route

R1

en

conf t

hostname R1

interface fa 0/0

no shutdown

ip address 192.168.3.2 255.255.255.0

exit

interface serial 2/0

no shutdown

clock rate 64000

ip address 192.168.4.1 255.255.255.0

end

show ip route

conf t

router ospf 1

network 192.168.3.0 0.0.0.255 area 0

network 192.168.4.0 0.0.0.255 area 0

end

show ip route

R2

en

conf t

hostname R2

interface fa 0/0

no shutdown

ip address 192.168.2.1 255.255.255.0

exit

interface serial 2/0

no shutdown

ip address 192.168.4.2 255.255.255.0

end

show ip route

conf t

router ospf 1

network 192.168.2.0 0.0.0.255 area 0

network 192.168.4.0 0.0.0.255 area 0

end

show ip route

PC1 Ping PC2

Ping 192.168.2.2   reply

补充实验结果图

思考问题：

- OSPF协议的路由决策依据是什么？